Gesture-Controlled Game Controller (Wired)

# 1.  Project

This project is a motion-based game control system that detects the user’s hand movements and converts them into in-game commands. The MPU6050 sensor used in the system continuously measures tilt and acceleration data. This data is processed by the Arduino Nano or ESP8266 and converted into meaningful commands, which are then sent to the computer via USB (Serial) connection. When the user tilts the device to the right or left, the character moves in that direction; when a sudden shaking motion is detected, actions such as an attack are triggered. On the computer side, this data is read by the Unity game engine and used directly as game input.

The aim of this project is to demonstrate how sensor data can be processed to convert physical movements into digital game control, and to reveal the connection between embedded systems and game development through a practical application.

Main component: MPU6050 sensor. This sensor contains two things accelerometer (acceleration) and a gyroscope.The accelerometer measures how fast you are moving, while the gyroscope measures the degree of rotation. By combining both, you can determine whether you are tilting or shaking.

# 2.  Materials

| Component | Qty | Price (TRY) | Purpose |
| --- | --- | --- | --- |
| Arduino Nano Clone | 1 | 130 TL | The brain of the project. It reads data from the sensor and sends it to the USB. |
| MPU6050 GY-521 | 1 | 120 TL | The sensor that detects hand movements. It measures acceleration and gyro simultaneously. |
| Breadboard (830 pin) | 1 | 45 TL | Allows us to establish connections without soldering. |
| Jumper Cable (M-F) | 1 set | 32 TL | Colored cables that connect the sensor to the Arduino. |
| USB Cable | 1 |  | Connects the Arduino to the computer and provides power at the same time. |

# 3.  How Are the Connections Made?

The MPU6050 sensor communicates using a protocol called I2C. For this, only 2 data cables are needed (SDA and SCL), plus power and ground. That makes 4 cables in total.

| MPU6050 Pin | Arduino Nano Pin | Description |
| --- | --- | --- |
| VCC | 5V | Power |
| GND | GND | Ground |
| SCL | A5 | I2C Clock (clock signal) |
| SDA | A4 | I2C Data (data line) |
| INT | D2 | Interrupt signal (optional) |

# 4.  Arduino

## 4.1  Calibration

When Arduino first starts, it reads the sensor 100 times and takes an average. This value becomes the reference point for "when the device is held flat." We subtract this from subsequent measurements to avoid unnecessary offset.

## 4.2  Roll and Pitch Calculation

We convert the raw values from the sensor into angles.

Roll gives left-right tilt, pitch gives forward-backward tilt.

## 4.3  Gesture Detection

If the angle is greater than 15 degrees, we say there is movement. You can adjust this threshold. It could be 10 degrees for someone with shaky hands, or 20.

| Command | Condition | In-Game Effect |
| --- | --- | --- |
| LEFT | roll < -15° | Character moves left |
| RIGHT | roll > +15° | Character moves right |
| FORWARD | pitch > +15° | Character moves forward |
| BACK | pitch < -15° | Character moves backward |
| SHAKE | total acceleration > 2.5G | Attack / jump triggered |
| BUTTON | D7 pin LOW | Interaction, item pickup, etc. |
| IDLE | None of the above | Character stops |

This check is performed every 50 milliseconds (i.e., 20 times per second) and the result is written to the Serial port. Unity reads this data and applies it to the character.

# 5.  Unity

## The Unity section consists of two scripts: SerialController.cs and GesturelPlayerController.cs

## 5.1  SerialController.cs

Handles communication between the computer and Arduino. It continuously reads the data coming through the Serial port and stores the most recent command.

Core responsibilities:

- Receiving text data from the Arduino

- Keeping this data in a processable format

- Making it accessible for other scripts to use

This script does not directly control game objects; it only works as a data reception layer.

## 5.2  GesturePlayerController.cs

This script reads the last command received by SerialController and applies it to the character in the game.
It causes the character to move or perform an action based on the incoming commands. For example:

- LEFT → move left

- RIGHT → move right

- ATTACK → trigger action

Thanks to this structure, the physical movements coming from the sensor are directly converted into game control within Unity.

.